# Q40 Fencing and Gates

PROJECT:

To be read with Preliminaries/ General conditions.

FENCING TYPE: RIDGEFENCE SPORTS REBOUND

FENCING

- Manufacturer: Ridgeway 103 Airport Road West, Belfast, BT3 9ED

Tel: 02890 454599 Fax: 02890 454596

E-mail: info@ridgeway-online.com www.ridgeway-online.com .

- All materials and installation to be covered by a 10year BEKassure Guarantee and to be subject to an inspection by an independent auditor on completion of works.  Licenced contractor to supply and install fencing (as indicated on drawing no              )

- PANELS: BEKASPORT PLUS (Varimesh Rebound System)

- Nominal Height: 2400mm / 3000mm / 4000mm / 5000mm / 6000mm

- Width panels: 2515mm

- Heights 3000mm and above is achieved using a combination of panels in a multilift configuration.

- Mesh and wire: Varimesh Welded Mesh spacing 50 x 66mm increasing to 50mm vertical x 200mm horizontal (above 1.2m) 6mm vertical and 8mm twin horizontal wires on the front and rear face of panel.  Galvanised & polyester powder coated Green RAL6005 / Black RAL9005.

- POSTS: BEKASPORT PLUS RHS

- Intermediate posts

- Galvanised rectangular hollow section, with M8 threaded inserts fitted in the front face, complete with 2D Super ‘U’ Channel clamp bar and M8 x 60 Countersunk Trilobe tamper proof bolts, posts and clamp bars polyester powder coated Green RAL6005 / Black RAL9005.

- Fixings: Panels secured to posts using a 2m custom ‘U’ Channel clamp bars.  Galvanised and polyester powder coated Green RAL6005 / Black RAL9005.

- Above 2m black 2D Super fixing brackets are used.

- Nominal centre’s of posts: 2.525m

- Corner posts

- Galvanised rectangular hollow section flanged corner post.  Polyester powder coated Green RAL6005 / Black RAL9005.

- Fixings: Panels secured to flanges using 2m custom ‘U’ Channel clamp bars.  Galvanised and polyester powder coated Green RAL6005 / Black RAL9005.

- Above 2m black 2D Super fixing brackets are used.

- GATES: ROBUSTA BALL COURT SWING GATE

Single leaf 1500mm wide x 3000mm high

- Gate to be galvanized & polyester coated Green RAL6005 / Black RAL9005 & clad with Nylofor F to match the fence.

- Infill to be fully welded to gate frame.

- Gate to be front hung complete with fully adjustable hinges opening outwards.

- Opening out of the Ballcourt.

- Gate complete with lockable ground bolts.

- Gate posts to have flanges to receive the adjoining panels.

- Gate with slip latch.

Double leaf 3000mm wide x 3000mm high

- Gate to be galvanized & polyester coated Green RAL6005 / Black RAL9005& clad with Nylofor F to match the fence.

- Infill to be fully welded to gate frame.

- Gate to be front hung complete with fully adjustable hinges opening outwards.

- Opening out of the Ballcourt.

- Gate complete with lockable ground bolts and Anti lift brackets.

- Gate posts to have flanges to receive the adjoining panels.

- Gate with slip latch.

- OTHER:

- Method of setting posts: In concrete foundations to comply with the design loading requirements specified by manufacturers technical specifications for this category of fence.

- Conformity:

- Sentinel Multi-Games satisfies the requirements for Category: 2 (Security) and Category: 3 (High Security) fences as prescribed by B.S. 1722 Part: 14 – Open Mesh Steel Panel Fences

- Upon receipt on site, or during storage off site, all care to be taken to ensure that no damage is incurred and that all precautions are taken against vandalism or theft.

- Materials and installation to be covered by a 10year BEKassure Guarantee.

- Subject to an inspection by an independent auditor on completion of works.

INSTALLATION

- Set out and erect fencing:

- As shown on drawings.

- Gradients

- Different gradients are achieved by stepping adjoining panels and/or reducing the post centres and width of panel giving a maximum gradient of 1 in 10 (10%).

- Steeper slopes will require extra long posts, manufactured with additional threaded inserts.

- With posts set rigid, plumb and to specified depth, or greater where necessary to ensure adequate support.

- Panels that are cut on site are to be made good using a zinc-etch primer paint, followed by a top coat of Cromadex 222 polyurethane repair paint.

- With correct fixings and all components securely fixed.

COMPETENCE

- The contractor shall employ experienced installers and provide qualified supervision to complete works as specified by manufacturer’s technical specifications for this category of fence.

- For information contact:

- Manufacturer: Ridgeway 103 Airport Road West, Belfast, BT3 9ED

Tel: 02890 454599 Fax: 02890 454596

E-mail: info@ridgeway-online.com www.ridgeway-online.com .

SETTING POSTS IN CONCRETE

- Holes: Excavate neatly and with vertical sides.

- Concrete mix: GEN1 or ST2 – to BS EN 206-1 using 20mm nominal size graded aggregates

- Alternative for small batch: the mix should not be leaner by mass than 1 part cement to 10 parts 20mm nom graded aggregates

- All concrete should be thoroughly compacted prior to the initial set.

- Exposed concrete foundations

- Filling: Compact until air bubbles cease to appear on the upper surface.

- Finishing: Weathered to shed water and trowelled smooth.